2

IOT Project: Keep An eye on it

Artevelde Hogeschool

IOT

Inhoudstafel

IOT Project	3

Arduino IDE	3

Controller	3

Hardware	3

PowerSupply	4

Components	5

Code: ESP32_Controller.ino	5

Eye Controller (Receiver)	10

Hardware	10

Code: ESP32_receiver.ino	10

Raspberry	15

RaspAP	15

Software	15

Add 2nd Wlan	16

3D Print	18

Controller	18

EyeSupport	19

# IOT Project

## Arduino IDE

Used Board

- ESP-WROOM-32

Download the ESP32 Board

https://dl.espressif.com/dl/package_esp32_index.json

Adds esp32 by Espressif Systems

## Cable Data Table

| VRX | VRY |  | SW | POT OUT |  | GND | 3V3 |  | GND | 5V |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Pot:

- 5V

- G32

- GND

Joystick

- SW: G4

- VRY: SN

- VRX: SP

- +5V: 3V3

- GND

Servo Controller:

- VCC: 3V3

- SDA: G21

- SCL: G22

- GNDµ

Servo’s:

- X: 0

- Y: 1

- LT: 2

- LB: 3

- RT: 4

- RB: 5

## Eye Controller

### Hardware

PowerSupply

### Components

| Li-Ion 18650 3400mAh |  |
| --- | --- |
| TP4056 |  |
| B10K |  |
| Joystick |  |
| ESP32-Wroom-32 |  |

### Code: ESP32_Controller.ino

#include <WiFi.h>

#include <PubSubClient.h>

#include <wifiSecret.h>

// WiFi credentials

const char* ssid = SECRET_SSID; // Enter your Wi-Fi name

const char* password = SECRET_PASS;  // Enter Wi-Fi password

const char* hostname = "ESP32-Controller";

// MQTT Broker

const char* mqtt_broker = "10.3.141.1";

const char* topic = "arduino/test";

const int mqtt_port = 1883;

// Pin definitions

#define POTENTIOMETER_PIN  32

#define JOYSTICK_SW_PIN    22

#define JOYSTICK_VRX_PIN   A0

#define JOYSTICK_VRY_PIN   A3

#define CLICK_INTERVAL 400 // Time interval for double click detection in milliseconds

#define DEBOUNCE_DELAY 50  // Debounce delay in milliseconds

WiFiClient espClient;

PubSubClient client(espClient);

int prevJoystickSwitchState = 1;

int prevJoystickXValue = -1;

int prevJoystickYValue = -1;

float prevPotentiometer = -1.0;

const int numSamples = 10;    // Number of samples to take

const float filterAlpha = 0.2; // Filter alpha for smoothing (0 < alpha < 1)

unsigned long lastDebounceTime = 0;

unsigned long lastClickTime = 0;

bool lastButtonState = HIGH;

bool buttonState = HIGH;

int clickCount = 0;

int currentJoystickState = 0;

void setup_wifi() {

delay(10);

Serial.println();

Serial.print("Connecting to ");

Serial.println(ssid);

// Stel de hostname in

if (!WiFi.setHostname(hostname)) {

Serial.println("Setting hostname failed!");

}

WiFi.begin(ssid, password);

unsigned long startAttemptTime = millis();

// Wait for connection with a timeout

while (WiFi.status() != WL_CONNECTED && millis() - startAttemptTime < 10000) {

delay(500);

Serial.print(".");

}

if (WiFi.status() != WL_CONNECTED) {

Serial.println("Failed to connect to WiFi");

// Optionally, you can restart the ESP32 here to try again

} else {

Serial.println("");

Serial.println("WiFi connected");

Serial.print("IP address: ");

Serial.println(WiFi.localIP());

}

}

void setup() {

// Initialize Serial Monitor

Serial.begin(115200);

// Initialize pins

pinMode(JOYSTICK_SW_PIN, INPUT_PULLUP);

pinMode(JOYSTICK_VRX_PIN, INPUT);

pinMode(JOYSTICK_VRY_PIN, INPUT);

// Connect to Wi-Fi

setup_wifi();

// Connect to MQTT broker

client.setServer(mqtt_broker, mqtt_port);

// Attempt to connect to the MQTT broker

while (!client.connected()) {

Serial.print("Connecting to MQTT...");

if (client.connect("ESP32Client")) {

Serial.println("ESP32Client connected");

} else {

Serial.println("failed with state ");

Serial.println(client.state());

delay(2000);

}

}

}

void loop() {

// Check if the Wi-Fi is still connected

if (WiFi.status() != WL_CONNECTED) {

Serial.println("WiFi disconnected. Attempting to reconnect...");

setup_wifi();

}

if (!client.connected()) {

// Reconnect to MQTT broker if connection is lost

while (!client.connected()) {

Serial.print("Reconnecting to MQTT...");

if (client.connect("ESP32Client")) {

Serial.println("connected");

} else {

Serial.print("failed with state ");

Serial.print(client.state());

delay(2000);

}

}

}

client.loop();

// Read the potentiometer value

int potentiometerValue = analogRead(POTENTIOMETER_PIN);

// Read the joystick values

float joystickXValue = analogRead(JOYSTICK_VRX_PIN);

float joystickYValue = analogRead(JOYSTICK_VRY_PIN);

int reading = digitalRead(JOYSTICK_SW_PIN);

// Debounce logic

if (reading != lastButtonState) {

lastDebounceTime = millis();

}

if ((millis() - lastDebounceTime) > DEBOUNCE_DELAY) {

if (reading != buttonState) {

buttonState = reading;

if (buttonState == LOW) { // Button pressed

if ((millis() - lastClickTime) < CLICK_INTERVAL) {

clickCount++;

} else {

clickCount = 1;

}

lastClickTime = millis();

}

}

}

lastButtonState = reading;

// Check for single or double click

if ((millis() - lastClickTime) > CLICK_INTERVAL && clickCount > 0) {

currentJoystickState = (clickCount == 1) ? 1 : 2;

clickCount = 0;

// Publish the joystick state

String joystickSwitchStateStr = "S: " + String(currentJoystickState);

client.publish(topic, joystickSwitchStateStr.c_str());

Serial.println(joystickSwitchStateStr);

}

// Reset the joystick state to 0 if the button is not pressed

if (buttonState == HIGH && currentJoystickState != 0) {

currentJoystickState = 0;

String joystickSwitchStateStr = "S: " + String(currentJoystickState);

client.publish(topic, joystickSwitchStateStr.c_str());

Serial.println(joystickSwitchStateStr);

}

// Convert and publish joystick X value if changed

if (joystickXValue != prevJoystickXValue) {

String joystickXValueStr = "X: " + String(joystickXValue);

client.publish(topic, joystickXValueStr.c_str());

Serial.println(joystickXValueStr);

prevJoystickXValue = joystickXValue; // Update previous state

}

// Convert and publish joystick Y value if changed

if (joystickYValue != prevJoystickYValue) {

String joystickYValueStr = "Y: " + String(joystickYValue);

client.publish(topic, joystickYValueStr.c_str());

Serial.println(joystickYValueStr);

prevJoystickYValue = joystickYValue; // Update previous state

}

// Publish potentiometer value if the average has changed significantly

if (potentiometerValue != prevPotentiometer) {

String potentiometerValueStr = "P: " + String(potentiometerValue);

client.publish(topic, potentiometerValueStr.c_str());

Serial.println(potentiometerValueStr);

prevPotentiometer = potentiometerValue; // Update previous average

}

// delay(200); // Small delay to prevent bouncing issues

}

float analogMultiSample(int pin) {

float sum = 0.0;

for (int i = 0; i < numSamples; i++) {

sum += analogRead(pin);

}

return sum / numSamples;

}

## Eye Receiver

### Hardware

| Servo |  |
| --- | --- |
| 16-channel 12-bits PWM driver I2C robot module |  |
| ESP32-Wroom-32 |  |

### Code: ESP32_receiver.ino

#include <Wire.h>

#include <Adafruit_PWMServoDriver.h>

#include <WiFi.h>

#include <PubSubClient.h>

#include <wifiSecret.h>

// WiFi credentials

const char* ssid = SECRET_SSID;

const char* password = SECRET_PASS;

const char* hostname = "ESP32-Receiver";

// MQTT Broker

const char* mqtt_broker = "10.3.141.1"; // IP of the Raspberry Pi running Mosquitto

const char* topic = "arduino/test";

const int mqtt_port = 1883;

// Adafruit PWM Servo Driver

Adafruit_PWMServoDriver pwm = Adafruit_PWMServoDriver();

const int servoEyeX = 0; // Servo connected to channel 0

const int servoEyeY = 1; // Servo connected to channel 1

const int servoLeftUp = 2; // Servo connected to channel 2

const int servoLeftDown = 3; // Servo connected to channel 3

const int servoRightUp = 4; // Servo connected to channel 4

const int servoRightDown = 5; // Servo connected to channel 5

WiFiClient espClient;

PubSubClient client(espClient);

// Servo min and max pulse lengths

const int SERVOMIN_X = 220; // Minimum pulse length count (out of 4096)

const int SERVOMAX_X = 440; // Maximum pulse length count (out of 4096)

const int SERVOMIN_Y = 250; // Minimum pulse length count (out of 4096)

const int SERVOMAX_Y = 500; // Maximum pulse length count (out of 4096)

unsigned long lastReconnectAttempt = 0;

void setup() {

Serial.begin(115200);

// Initialize the PWM driver

pwm.begin();

pwm.setPWMFreq(60);  // Analog servos run at ~60 Hz updates

// Connect to Wi-Fi

setup_wifi();

// Connect to MQTT broker

client.setServer(mqtt_broker, mqtt_port);

client.setCallback(callback);

connect_to_broker();

}

void setup_wifi() {

delay(10);

Serial.println();

Serial.print("Connecting to ");

Serial.println(ssid);

// Stel de hostname in

if (!WiFi.setHostname(hostname)) {

Serial.println("Setting hostname failed!");

}

WiFi.begin(ssid, password);

unsigned long startAttemptTime = millis();

// Wait for connection with a timeout

while (WiFi.status() != WL_CONNECTED && millis() - startAttemptTime < 10000) {

delay(500);

Serial.print(".");

}

if (WiFi.status() != WL_CONNECTED) {

Serial.println("Failed to connect to WiFi");

// Optionally, you can restart the ESP32 here to try again

} else {

Serial.println("");

Serial.println("WiFi connected");

Serial.print("IP address: ");

Serial.println(WiFi.localIP());

}

}

// Attempt to connect to the MQTT broker

void connect_to_broker() {

while (!client.connected()) {

Serial.print("Connecting to MQTT...");

if (client.connect("ESP32Subscriber")) {

Serial.println("ESP32Subscriber connected");

client.subscribe(topic);

} else {

Serial.print("failed with state ");

Serial.print(client.state());

delay(2000);

}

}

}

void callback(char* topic, byte* payload, unsigned int length) {

// Serial.print("Message arrived [");

// Serial.print(topic);

// Serial.print("] ");

String message;

for (int i = 0; i < length; i++) {

message += (char)payload[i];

}

// Serial.println(message);

// Horizontale oogbeweging

int ColIndexX = message.indexOf("X:");

if (ColIndexX != -1) {

String ValueMessageX = message.substring(ColIndexX + 2); // Skip "X:"

int angleX = ValueMessageX.toInt(); // Scale factor

int pulseX = map(angleX, 0, 4095, SERVOMIN_X, SERVOMAX_X);

pwm.setPWM(servoEyeX, 0, pulseX);

Serial.print("Servo angleX set to: ");

Serial.println(angleX);

}

// Verticale oogbeweging

int ColIndexY = message.indexOf("Y:");

if (ColIndexY != -1) {

String ValueMessageY = message.substring(ColIndexY + 2); // Skip "Y:"

int angleY = ValueMessageY.toInt(); // Scale factor

int pulseY = map(angleY, 0, 4095, SERVOMIN_Y, SERVOMAX_Y);

pwm.setPWM(servoEyeY, 0, pulseY);

Serial.print("Servo angleY set to: ");

Serial.println(angleY);

}

// // Oogleden

// int ColIndexP = message.indexOf("Potentiometer Value:");

// if (ColIndexP != -1) {

//   String ValueMessageP = message.substring(ColIndexP + 19); // Skip "Potentiometer Value:"

//   int angleP = ValueMessageP.toInt();

//   if (angleP >= 0 && angleP <= 180) {

//     int pulseP = map(angleP, 0, 180, SERVOMIN_X, SERVOMAX_X);

//     pwm.setPWM(servoLeftUp, 0, pulseP);

//     pwm.setPWM(servoLeftDown, 0, 180-pulseP);

//     pwm.setPWM(servoRightUp, 0, pulseP);

//     pwm.setPWM(servoRightDown, 0, 180-pulseP);

//     Serial.print("Servo angleP set to: ");

//     Serial.println(angleP);

//   } else {

//     Serial.println("Invalid angle for Potentiometer Value");

//   }

// }

}

void loop() {

// Check if the Wi-Fi is still connected

if (WiFi.status() != WL_CONNECTED) {

Serial.println("WiFi disconnected. Attempting to reconnect...");

setup_wifi();

}

// Check if the MQTT client is still connected

if (!client.connected()) {

unsigned long now = millis();

if (now - lastReconnectAttempt > 5000) {

lastReconnectAttempt = now;

// Attempt to reconnect

connect_to_broker();

}

} else {

// Client connected, reset last reconnect attempt time

lastReconnectAttempt = 0;

}

client.loop();

}

## Raspberry

### RaspAP

### Software

MQTT Broker

### Add 2nd Wlan

Om de ASUS USB-N13 C1 draadloze adapter op een Raspberry Pi te installeren, moet je een aantal stappen volgen. Hieronder staat een gedetailleerde handleiding:

Voorbereiding

Zorg ervoor dat je Raspberry Pi is bijgewerkt met de laatste software.

Open een terminal en voer de volgende commando's uit om je systeem bij te werken:

bash

Code kopiëren

sudo apt update

sudo apt upgrade

Installatie van vereiste pakketten

Installeer de vereiste pakketten voor het bouwen van stuurprogramma's:

bash

Code kopiëren

sudo apt install dkms build-essential libelf-dev linux-headers-$(uname -r)

Download de stuurprogramma's

Ga naar de GitHub-pagina waar de stuurprogramma's voor de ASUS USB-N13 C1 beschikbaar zijn. Je kunt bijvoorbeeld de Realtek 8812au driver gebruiken:

bash

Code kopiëren

git clone https://github.com/aircrack-ng/rtl8812au.git

Build en installeer de stuurprogramma's

Navigeer naar de directory van de gedownloade stuurprogramma's en bouw deze:

bash

Code kopiëren

cd rtl8812au

sudo make dkms_install

Controleren van de installatie

Na het installeren van de stuurprogramma's, moet je de USB-N13 C1 adapter aansluiten op je Raspberry Pi. Controleer of de adapter correct is geïnstalleerd door het volgende commando uit te voeren:

bash

Code kopiëren

lsusb

Je zou iets moeten zien zoals "ASUS USB-N13 802.11n Network Adapter".

Netwerkconfiguratie

Gebruik raspi-config om de Wi-Fi in te stellen:

bash

Code kopiëren

sudo raspi-config

Ga naar "Network Options" en vervolgens "Wi-Fi" om je Wi-Fi-netwerk te configureren.

Herstart je Raspberry Pi

Na het instellen van de Wi-Fi moet je de Raspberry Pi opnieuw opstarten:

bash

Code kopiëren

sudo reboot

Na het herstarten zou je Raspberry Pi verbinding moeten kunnen maken met Wi-Fi via de ASUS USB-N13 C1 adapter. Als je problemen tegenkomt, kun je de logbestanden controleren voor meer details:

bash

Code kopiëren

dmesg | grep 8812au

Dit zou je een gedetailleerde gids moeten geven om de ASUS USB-N13 C1 draadloze adapter op je Raspberry Pi te installeren.

## 3D Print

Hardware Ender 3V3

### Controller

Base

Top

### EyeSupport